SYSTEM REQUIREMENTS

Real-Time Pan-Tilt Laser Tracking System

| Author | Duncan Nam |
| --- | --- |
| Version | 1.1 |
| Date | 2026-MAY-09 |
| Status | Draft |

# 1. System Overview

The system shall detect a user-defined colored object in real time using a USB webcam, compute the object's centroid position in the camera frame, and command a two-axis pan-tilt servo mount to center the object. A laser pointer mounted to the pan-tilt assembly shall track the object. The live camera feed shall be accessible from a MacBook Air over the local WiFi network. The system is intended as a personal engineering demonstration project.

# 2. Functional Requirements

- FR-01  Color Detection: System shall detect a user-specified HSV color range in the camera frame and identify the target object's centroid.

- FR-02  Servo Control: System shall command pan and tilt servo angles to maintain target centroid within a defined deadband of frame center.

- FR-03  PID Tracking: System shall implement a proportional-integral-derivative (PID) control loop to drive pan/tilt error to zero.

- FR-04  Serial Link: Raspberry Pi shall transmit pan/tilt angle commands to Arduino via USB serial.

- FR-05  Video Stream: System shall stream the annotated live camera feed (with centroid overlay) to a MacBook Air via local WiFi network browser.

- FR-06  Laser Output: A 5mW 650nm laser module mounted to the servo assembly shall point at the tracked object.

- FR-07  Camera Mount: Camera shall be co-mounted on the pan-tilt servo assembly with the laser module and move as a single rigid unit.

# 3. Performance Requirements

- PR-01  Frame Rate: Vision processing pipeline shall operate at >= 30 fps.

- PR-02  Tracking Latency: End-to-end latency from object movement to servo response shall be <= 150 ms.

- PR-03  Angular Range: Pan axis: +/-90 degrees. Tilt axis: +/-45 degrees.

- PR-04  Deadband: Servo shall not be commanded if centroid error is < 5 pixels from frame center (prevents jitter).

- PR-05  Stream Latency: Video stream latency to MacBook browser shall be <= 500 ms.

# 4. Hardware Constraints

- HC-01  Compute: Raspberry Pi 5.

- HC-02  Microcontroller: Arduino Uno for servo PWM output.

- HC-03  Camera: USB webcam (e.g., Logitech C920), connected to Raspberry Pi.

- HC-04  Servos: 2x MG996R Metal Gear servos on aluminum pan-tilt bracket. MG996R selected over MG90S after torque analysis — camera co-mount load (~200g) requires minimum 0.8kg/cm; MG996R provides 9kg/cm at 4.8V with adequate safety margin.

- HC-05  Laser: 5mW 650nm laser module, 3.3V or 5V compatible.

- HC-06  RPi-Arduino Link: USB serial (via termios.h).

- HC-07  Servo Power: Dedicated 5V 5A DC power supply for servo power domain. Servo power isolated from Arduino and RPi power domains. Common ground shared across all domains.

# 5. Software Constraints

- SC-01  Language: C++ on Raspberry Pi. C++ (Arduino framework) on Uno.

- SC-02  Vision Library: OpenCV C++ API.

- SC-03  OS: Raspberry Pi OS (64-bit, Debian-based Linux).

- SC-04  Streaming: mjpg-streamer running as a background process on RPi.

- SC-05  Version Control: Git / GitHub with README and demo video for portfolio visibility.

# 6. Out of Scope

- Multi-object tracking.

- Autonomous target classification (color heuristic only, no ML).

- Wireless Arduino communication.

- Persistent data logging.

# 7. Acceptance Criteria

The system shall be considered complete when it can reliably track a handheld colored object across the full angular range of the mount for >= 30 seconds without losing lock, with the annotated feed simultaneously visible in a MacBook Air browser.

# 8. Future Improvements

The following capabilities are planned for future versions and are out of scope for v1.0:

- Variable lighting adaptation via adaptive HSV thresholding or histogram equalization.

- Multi-object tracking with target prioritization.

- Stereo vision range finding using a second Logitech Brio 101 at a fixed baseline.

- LiDAR module (e.g. TF-Luna) integration for precision ranging at extended distances.

- Upgrade from proportional-only control to full PID controller with integral and derivative terms.

# 9. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-MAY-02 | Duncan Nam | Initial release. |
| 1.1 | 2026-MAY-09 | Duncan Nam | Updated servo selection to MG996R, added FR-07 camera mount requirement, added HC-07 servo power constraint, added Section 8 Future Improvements, added revision history. |

# Appendix A — System Architecture Diagram

Figure A-1. Hardware and software component interfaces.

v1.1 — Duncan Nam — 2026-MAY-09